# Laboratory 15 - Introduction to the SDK-8085 Microcomputer

Goals:

- Understand the fundamental hardware and software structure of a microcomputer.

- Program and execute some machine language programs.

Background:

The SDK-8085 (the SDK) is a single board microcomputer with an 8085 CPU.  This is the grandfather of the 8088, 80286, 80386, 80486, and PENTIUM processors used in IBM and compatible computers.  It has an eight bit wide data bus and sixteen bit wide address bus. The following details the hardware memory design of the SDK.

Memory Map for the SDK-8085 with NVRAM and EPROM addition.

The memory is decoded into eight 2K byte blocks at two different places in memory.  Sixteen kilobytes of memory is one quarter of the whole memory addressing space.  The four ranges are:

Hex Address		Hex Address		Comments

0000H			3FFFH			The SDK area

4000H			7FFFH			Not decoded

8000H			BFFFH			NVRAM and EPROM area

C000H			FFFFH			Not decoded

A closer look at the lowest 16K addresses yields:

Hex Address		Hex Address		Comments

0000H			07FFH			Monitor Rom (2K)

0800H			0FFFH			Expansion Rom (not used)

1000H			17FFH			Not used

1800H			1FFFH			Keybd/Display Control

2000H			27FFH			RAM (1/4 Kbyte folded over

eight times)

2800H			2FFFH			      "

3000H			37FFH			Not used

3800H			3FFFH			Not used

A closer look at the third 16K block of memory:

Hex Address		Hex Address		Comments

8000H			87FFH			NVRAM (eventually 2K, now

1/2 K 8600-87FF)

8800H			8FFFH			Not used

9000H			97FFH			EPROM (2K  2716 type)

9800H			9FFFH			Not used

A000H			A7FFH			Not used

A800H			AFFFH			Not used

B000H			B7FFH			Not used

B800H			BFFFH			Not used

Each of the "Not used" 2K blocks of memory has a Chip Select pulse available.  These are from the 8205 decoder on the main board and the 74138 (identical to the 8205) on the expansion area.  For a description of this decoding, see the "MCS - 80/85 Family User's Manual" and the "SDK - 85 System Design Kit User's Manual."

Appendix E is a listing of some of the instructions available in the 8085 CPU instruction set.  There are one, two, and three byte instructions available for your use.  The instruction allow the movement of data from register to register, between memory and registers, math, logic, etc.

Appendix F is a list of available programs (they start with PR) and subroutines (they start with SR).  The programs are stand alone.  They are used for various experiments and for certain system operations. The subroutines are to be used within other programs.  They do such tasks as displaying to the display, reading the keyboard, random number generator, etc.

Procedure:

1.) You are going to look at the decoding scheme for the SDK.  The decoding is done by a 74138 decoder (an 8205 or 3205) on the computer.  See Appendix C for more information about this IC.  It is directly above the 8085 CPU.  It is a 3 to 8 decoder with three enables, one high , two low.  It is wired as shown below

The A15 to A0 lines are the sixteen address lines.  (1) In hexadecimal notation, give the range of addresses which will activate each of the outputs O0 to O7.  (2) Do your values agree with the  values from the previous memory map?

2.)  Turn the computer on by turning on the blue power supply. The basic operation of the SDK-8085 is quite simple.  The four operations we will use are:  RESET, EXAMINE MEMORY, SUBSTITUTE MEMORY, and GO.  Key strokes entered from the keypad will be given by <  >, and the contents of the display will be given by (  ).

RESET

The RESET key on the keypad will cause the computer to "start over."  The current contents of the RAM memory are not lost, but any program execution is halted and the monitor program starts over.  This RESET is a "hardware reset".  The message "-8085" will be displayed in the display just as it is when the power is turned on.

<RESET>		(-8085)

Used for breaking out of a program or reinitializing the system.

EXAMINE MEMORY CONTENTS

The contents of any memory location, RAM or ROM, may be examined.  For example, to examine the contents of memory location 92ACH push the following keystroke sequence.

<RESET>		( - 80 85 )

<SUBST MEM>	( . )

< 9 >			( 0 0 0 9 )

< 2 >			( 0 0 9 2 )

< A >			( 0 9 2 A )

< C >			( 9 2 A C )

< NEXT >		( 9 2 A C    0 7 )

After the NEXT key is pushed, the 0 and 7 in the data field of the display will be the data (in Hex numbers) of the memory address 92ACH.  This address is shown in the address field of the display.

If the NEXT button is pushed again, the address 92ACH is incremented by one and the contents of this new memory locations shown in the data field of the display.  This may be repeated as many times as you like.  You cannot go backwards to smaller numbers.

To escape, you may push either the RESET or EXEC buttons.

The RESET starts the monitor over again, and the EXEC button causes an escape from the Examine Memory sequence and waits for the next command from the keypad.

CHANGE MEMORY CONTENTS

To change the contents of a memory location the same sequence as above is used, but after the NEXT is pushed the next two keystrokes will appear in the data field of the display.  If NEXT is now pushed, this new data is put into the memory location just examined and the address and the contents of the next memory are displayed.  Again, two keystrokes from the keyboard followed by the NEXT button will cause this data to be entered into the memory.  Only pushing NEXT or EXEC will cause the new data entered from the keypad to be written into the memory.  The EXEC will cause an escape from the examine memory sequence and so will the RESET.  However, the RESET will not write the last data change into memory.

PROGRAM EXECUTION

The following sequence will cause a program to execute starting at a memory location of your  choice.

< GO >		( . )

< Hex num 1 >	( 0 0 0 n1 )

< Hex num 2 >	( 0 0 n1 n2 )

< Hex num 3 >	( 0 n1 n2 n3 )

< Hex num 4 > 	( n1 n2 n3 n4 )

<  EXEC >		( E )

The computer will begin the execution of your program at the memory address n1 n2 n3 n4.  The "E" in the display indicates the computer is executing a program.  Unless your program eventually jumps to the monitor restart point, the only way out of the program execution is the RESET button.

a.)	Examine memory location 07D1 (generally written 07D1H where the H indicated that the number preceding it is a Hex number.)  (3) What number is at this location?

b.)	Change the contents of RAM memory location 2050H to D9H and then check to see that the change has been made. (4) How do you do this checking and what did you find?

c.)	Start program execution at location 0000H.  This is what the RESET button does. (5) Agree?

3.)  Enter and run the following programs.  Be sure to understand how each statement contributes to the program.  These programs are to introduce machine code in a step by step fashion.

PROGRAM 1 - Load the accumulator and move it to a memory location.

Addr		MC		Assem		Comments

8600		31		LXI SP		;SETUP STACK POINTER

8601		C2		C2

8602		20		20

8603		3E		MVI A		;LOAD ACCUM WITH FOLLOWING    BYTE

8604		35		35			;ANY NUMBER

8605		21		LXI H		;LOAD HL PAIR WITH THE         FOLLOWING

8606		E4		E4			;      L = E4

8607		28		28			;      H = 28

8608		77		MOV M,A		;MOVES ACCUM TO MEMORY LOCATION POINTED TO BY HL REG. CONTENTS IN THIS CASE, 28E4

8609		76		HALT			;STOPS EXECUTION

1.)	Load memory location 28E4 with 00 from the keyboard using the SUBST MEMORY key sequence.

2.)	Check it to make sure it is there.

3.)	Load the program using the SUBST MEMORY key sequence.

4.)	Run the program using the GO key sequence.

5.)	Check the contents of 28E4 and verify that the program did indeed write 35 to memory location 28E4.

(6) How would you change the program to write a different number to this memory location?

(7) How would you change the memory location to which the accumulator is stored?

PROGRAM 2 - Displays the accumulator to the data field of the display.

Addr		MC		Assem		Comments

8600		31		LXI SP		;SET UP STACK POINTER

8601		C2		C2

8602		20		20

8603		3E		MVI A			;LOAD ACCUM. WITH FOLLOWING BYTE

8604		42		42			;BYTE TO MOVE INTO ACCUM

8605		CD		CALL			;CALL TO SUBROUTINE, THE

8606		C0		C0			ADDRESS IS GIVEN IN THE TWO

8607		90		90			BYTES FOLLOWING THE INSTRUCTION LOW BYTE OF ADDRESS FIST

8608		76		HLT			;WHERE THE COMPUTER RETURNS FROM THE SUBROUTINE AND ENDS EXECUTION

1.)	Load the program and verify its operation.

(8) How would you display a different number to the data field?

PROGRAM 3 - A counter in the address field of the display.

Addr		MC		Assem		Comments

8600		31		LXI SP		;SET UP STACK POINTER

8601		C2		C2

8602		20		20

8603		11		LXI D			;LOAD DE REGS WITH FOLLOWING

8604		00		00				E = 00

8605		00		00				D = 00

8606		CD           L1:	CALL			;SUBROUTINE FOR DISPLAY OF DE

8607		B0		B0				IN THE ADDRESS FIELD OF

8608		90		90				OF THE DISPLAY

8609		13		INX D			;ADDS ONE TO THE 16 BIT REG MADE UP OF DE TOGETHER

860A		C3		JMP L1		;UNCONDITIONAL JUMP TO

860B		06		06			FOLLOWING LOCATION LABELED L1

860C		86		86			;ENDLESS LOOP

Load and execute the program.

(9) Why is it going so fast?

PROGRAM 3a - Slowing the counter down.

Leave everything up to 860A alone and add the following.

860A		D5		PUSH D		;SAVES DE ON THE STACK

860B		11		LXI D			;LOAD DE AGAIN WITH NUMBER FOR

860C		FF		FF				DELAY LOOP.  FFFF IS THE

860D		FF		FF				LONGEST AND 0001 SHORTEST

860E		CD		CALL			;CALL TO DELAY ROUTINE

860F		D0		D0				COUNTS DOWN THE CONTENTS

8610		90		90				OF THE DE REG PAIR

8611		D1		POP D		;RESTORES DE (YOUR COUNTER)

FROM THE STACK

8612		C3		JMP L1		;JUMP BACK TO THE LOOP L1

8613		06		06				NEVER ENDING LOOP FROM

8614		86		86				8606 (L1) TO HERE

Load this addition and execute the program.

(10) Did it slow it down?

(11) Change the contents of the delay subroutine (change DE  regiser pair before the CALL) and verify that this does change the counting rate.

(12) What hex number is half the size of FFFF?

PROGRAM 4 - Fill in a page of memory with zeros.

Addr		MC		Assem		Comments

8600		31		LXI SP		;SET UP STACK

8601		C2		C2

8602		20		20

8603		21		LXI HL		;PICK PAGE OF MEMORY FOR

8604		00		00			ZEROING I PICK THE EXPANSION

RAM

8605		28		28

8606		AF		XRA A		;EXCLUSIVE ORS ACCUM WITH ACCUM ZEROS ACCUM IN ONE STATEMENT

8607		77	 LP:	MOV M,A		;MOVES ZERO TO FIRST MEM LOCATION

8608		2C		INR L			;INCREMENTS L

8609		C2		JNZ LP		;JUMPS IF L DID NOT SET THE ZERO FLAG

860A		07		07

860B		86		86

860C		CF		RST1			;INTERRUPTS BACK TO MONITOR

1.)	Look at 2800 page and see if it contains zeros before you start.

2.)	Load and run the program.

3.)	Look at 2800 again and see if it contains zeros.

(13) Did it work? (14) What does the commmand at location 8606 do?  (15) How would you change the program to load the 2800 page with something else?

(16) How would you change the page you wished to fill with something?

PROGRAM 5 - A quick look at the IN instruction.

Addr		MC		Assem		comments

8600		31		LXI SP		;SET UP STACK

8601		C2		C2

8602		20		20

8603		DB		IN			;INPUT FROM PORT ADDRESS OF

8604		21		21				THE NEXT CHANNEL (21 IN THIS CASE)

8605		CD		CALL			;SUBROUTINE TO DISPLAY THE

8606		CO		CO				ACCUMULATOR

8607		90		90

8608		C3		JMP			;UNCONDITIONAL JUMP BACK TO

8609		03		03				THE IN INSTRUCTION WHICH

860A		86		86				LOOPS OVER AND OVER

1.)	Load and execute this program.  Touch Port 21 on the SDK -8085 and watch the display.  The ports are along the right edge of the wire wrap area of the computer.  Port 21 (and 22 and 23) have a header in them (little metal posts sticking up).  (17) Does it work?

2.)	Touch other Ports and watch the display.  They should not have as much influence on the state of the display as Port 21 does.  (18) Why?

3.)	Change the program to input from Port 2B, find it on the computer and verify that you are now inputing from this Port and no longer from Port 21.  (19) How can you tell?

(20) How many different Input Ports are available?

PROGRAM 6 - Output ports.

Addr		MC		Assem		Comments

8600		31		LXI SP		;SET UP STACK

8601		C2		C2

8602		20		20

8603		3E		MVI A			;LOAD STATUS CONTROL REGISTER

8604		01		01				WITH 01 WHICH WILL MAKE

PORT 21 AN OUTPUT PORT

8605		D3		OUT			;OUTPUTS 01 TO PORT 20 THE

8606		20		20				COMMAND STATUS REGISTER

FOR PORTS 21, 22 AND 23

NOTE: 22 AND 23 ARE STILL INPUT PORTS!

8607		3E		MVI A			;LOAD A WITH ALL ZEROS IN THE

HIGHEST FOUR BITS AND ONES IN THE LOWEST FOUR BITS

8608		OF		OF

8609		D3		OUT			;OUTPUT THIS BIT PATTERN TO

860A		21		21				PORT 21

860B		76		HLT			;STOP EXECUTION

1.)	Run the program and use the logic probe to determine whether you have put the correct bit pattern out to Port 21.  (21) Agree? Look at other ports with the logic probe.  Since they are input ports (the default for this computer) they are high impedance and thus are not high or low. (22) Agree?

2.)	Hit the RESET button and check Port 21 again.  You will find if you check with the logic probe that it is now back to an input port.  (23) Agree?  The monitor program does this on RESET.

(24) How would you make both Ports 21 and 22 output ports?  Try this and put out two different numbers to these ports and check them with your logic probe.   Map the bit order at the Port 21 by setting one bit at a time and seeing to which pin this corresponds.  (25) What numbers (hex and decimal) set each of the eight bits?